| DATE : | January 3, 2006 |
| --- | --- |
| TO : | Howard Markus, Research Scientist Biological Monitoring Unit Water Assessment and Environmental Information Section Environmental Analysis and Outcomes Division |
| FROM : | Steven Heiskary, Research Scientist Lakes and Streams Unit Water Assessment and Environmental Information Section Environmental Analysis and Outcomes Division |
| PHONE : | (651) 296-7217 |
| SUBJECT : | 2006 TMDL Nutrient-Impaired Lake List: Update |

On June 7, 2005 I provided you with the draft list of nutrient impaired lakes for 2006.  As a part of that I included a memorandum that provided some background on my data review process and a brief discussion for each lake that served to support its inclusion or exclusion from the 2006 draft list.  As noted in that discussion there were some lakes that may be excluded from the final list as a result of public comment, new data, or further analysis of existing data.  Also there may be a few lakes added to the list as a result of data that was previously collected during the assessment period (1995-2004) but not in STORET at the time of the data retrieval for the 303(d) assessment.  Rather than redoing the original memorandum I will use this “Update” to provide new or updated information, assessments or comments I receive as a part of the public review process.  The notes I make herein will allow you to amend the draft list as needed.   I will do this as a running list and include future edits within the context of this memorandum.

1. Sarah Lake (27-0191) is a 574 acre lake with a maximum depth of 59 feet and a littoral area that comprises 65% of the lake.  It is located southeast of Rockford in Hennepin County.  For data storage and assessment purposes the lake was recently bayed as follows: West Sarah Lake (27-0191-01) and East Sarah Lake (27-0191-02)

While it had an extensive CLMP database it was lacking TP and chlorophyll-a data; however recently data from the Three Rivers Park District was entered into STORET and now the lake has sufficient data for a 303(d) assessment.  Since the data was collected during the 1995 – 2004 (1996-2002) timeframe an assessment for the 2006 list was conducted.

As noted below the West basin (outlet bay) exceeds the NCHF ecoregion thresholds for TP and chlorophyll-a and is only slightly better than the Secchi threshold.  The East Basin lacks the TP and chlorophyll-a measurements for the assessment period; however the Secchi is very low at 1.1 meters and the long-term Secchi record indicates several summers with summer-mean Secchi less than 1.1 meters.  In 1991 data were collected in both bays and at that time TP average 120 ppb in the East bay and 130 ppb in the West.  Given the data record for both bays and their interconnectedness (considered a single lake by MDNR) I would recommend listing the entire lake on the 2006 list.

| Station_ID | Lake | Bas | COUNTY | Acre | Depth - max | TP | NP | Chl-a | NC | SD | NS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27-0191-01 | West Sarah | UM | Hennepin |  |  | 105 | 38 | 42 | 37 | 1.2 | 50 |
| 27-0191-02 | East Sarah | UM | Hennepin |  |  |  |  |  |  | 1.1 | 110 |
| 27-0191 | Sarah (whole) | UM | Hennepin | 574 | 59 |  |  |  |  |  |  |

2. McDonald (82-0010) and Downs  (82-0110) Lakes  - Question was raised by Dr. Hal Runke, on behalf of Valley Branch Watershed District, as to whether these two waterbodies should be considered “lakes” for the purpose of 303(d) assessment since they both had the “LW” classification in the PWI.  I had raised this as a potential issue for McDonald Lake in my June 7, 2005 memorandum.  I had asked for a letter from the District describing their concern regarding this but have not yet received anything on this.  If the two are indeed classified as “wetlands” in the PWI they should be removed from the draft list; absent someone providing information to the contrary.

## 3. Data for a lake listed as 82-0086 at “Glendenning Avenue” was in actuality collected from 82-0087, which is referred to as “Unnamed or Ravine Lake in Cottage Grove Ravine Park (Ravine Lake based on MDNR fisheries survey)” based on a call from Randy Anhorn at MCES (data collected through CAMP program).  Its status in the PWI is unknown at this point; however MDNR had the following entry on Lake Finder:

## Status of the Fishery (as of 08/08/2001) “Ravine Lake is located in Cottage Grove Ravine Regional Park. It was netted for the first time in 2001 to get current fish population information. The water level in Ravine Lake has been rising in recent years - creating more deep water. Current lake size is 25 acres and maximum depth is 16 feet. The lake is fringed with flooded timber. There is no formal access, but carry-in access is possible from the parking lot. Limited shore fishing is also available.

Black bullhead was the most abundant species sampled. Lengths ranged from 7 to 10 inches, with an 8 inch average. Walleye were sampled in moderate numbers. Most fish were between 15 and 20 inches. The walleyes are fish that survived from a fry stocking in 1995, when the lake was used as a DNR rearing pond. One largemouth bass was sampled - it was 19 inches long and weighed 5.25 pounds. Green sunfish were abundant - lengths ranged from 1.5 to 6.5 inches.”

Given this information it would appear that this unnamed water body is a lake and based on the previous data summarized in my June 7, 2005 memorandum the lake should be listed.